Exp 1: analyses and development and verification of 8 bit and 16 bit arithmetic operations using assembly language program

OBJECTIVES

- To analyse and verify 8-bit addition and subtraction using an 8-bit microprocessor.

- To analyse and verify 16-bit addition and subtraction using an 8-bit microprocessor.

- To analyse and verify 8-bit multiplication and division using an 8-bit microprocessor.

- To analyse and verify 16-bit multiplication and division using an 8-bit microprocessor.

Components required:

8085 simulator

Procedure:

1.Switch ON the microprocessor kit.

2.Enter the starting memory address of the program.

3.Enter the required 8085 assembly language instructions one by one.

4.Enter the input data at the specified memory locations.

5.Execute the program from the starting address.

6.Check the output stored in the specified memory location.

7.Verify the result with the expected output.

8.Switch OFF the microprocessor kit after completing the experiment.

1. (i) To analyse and verify 8-bit addition using 8  bit microprocessor.

PROGRAM:

MVI C 00h

LDA 4200h

MOV B A

LDA 4201h

ADD B

JNC SKIP

INR C

SKIP: STA 4202h

MOV A C

STA 4203h

HLT

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 4200 | FF |
| 4201 | FF |
| 4202 | FE |
| 4203 | 01 |

(ii) To analyse and verify 8-bit subtraction using 8  bit microprocessor.

PROGRAM:

MVI C 00h

LDA 4201h

MOV B A

LDA 4200h

SUB B

JNC SKIP

CMA

INR A

INR C

SKIP: STA 4202h

MOV A C

STA 4203h

HLT

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 4200 | AB |
| 4201 | 3A |
| 4202 | 71 |
| 4203 | 00 |

RESULT:

Thus, 8-bit addition and subtraction were performed and verified successfully using the 8085 microprocessor.

2. (i)   To analyse and verify 16-bit addition using  8 bit microprocessor

PROGRAM:

MVI C 00h

LHLD 4000h

XCHG

LHLD 4202h

DAD D

JNC SKIP_CARRY

INR C

SKIP_CARRY: SHLD 4004

MOV A C

STA 4006h

HLT

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 4000 | D5 |
| 4001 | F8 |
| 4002 | CD |
| 4003 | D5 |
| 4004 | A2 |
| 4005 | CE |
| 4005 | 01 |

(ii) To analyse and verify 16-bit subtraction using  8bit microprocessor

PROGRAM:

MVI C 00

LHLD 4000

MOV A L

LHLD 4002

SUB L

MOV L A

MOV A H

SBB H

MOV H A

JNC SKIP_BORROW

INR C

CMA

MOV H A

MOV A L

CMA

ADI 01

MOV L A

MOV A H

ACI 00

MOV H A

SKIP_BORROW: SHLD 4004

MOV A C

STA 4006

HLT

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 4000 | D5 |
| 4001 | F8 |
| 4002 | CD |
| 4003 | D5 |
| 4004 | 08 |
| 4005 | 23 |
| 4006 | 00 |

RESULT:

Thus, 16-bit addition and subtraction were performed and verified successfully using the 8085 microprocessor.

3. (i)   To analyse and verify 8-bit multiplication using  8 bit microprocessor

PROGRAM:

LDA 4200h

MOV E A

LDA 4201h

MOV B A

LXI H 0000h

MVI D 00h

NEXT: DAD D

DCR B

JNZ NEXT

SHLD 4202h

HLT

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 4200 | 2A |
| 4201 | 03 |
| 4202 | 7E |
| 4203 | 00 |

(ii) To analyse and verify 8-bit division using  8 bit microprocessor.

PROGRAM:

LXI H 2050h

MOV A M

INX H

MOV B M

MVI C 00h

LOOP: CMP B

JC DONE

SUB B

INR C

JMP LOOP

DONE: STA 2052h

MOV A C

STA 2053h

HLT

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 2020 | 14 |
| 2051 | 04 |
| 2052 | 00 |
| 2053 | 05 |

RESULT:

Thus, 8-bit multiplication and division were performed and verified successfully using the 8085 microprocessor.

4.(i) To analyse and verify 16bit multiplication  using  8bit microprocessor

PROGRAM:

LXI H 2050h

MOV E M

INX H

MOV D M

LXI H 2052h

MOV C M

INX H

MOV B M

LXI H 0000h

LOOP: DAD D

DCX B

MOV A B

ORA C

JNZ LOOP

SHLD 2054h

HLT

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 2050 | 05 |
| 2051 | 00 |
| 2052 | 03 |
| 2053 | 00 |
| 2054 | 0F |
| 2055 | 00 |

(ii) To analyse and verify 16-bit division  using  8bit microprocessor.

PROGRAM:

LXI H 2050h

MOV E M

INX H

MOV D M

LXI H 2052h

MOV C M

INX H

MOV B M

MOV A B

ORA C

JZ STOP

LXI H 0000h

LOOP: MOV A E

SUB C

MOV A D

SBB B

JC DONE

MOV A E

SUB C

MOV E A

MOV A D

SBB B

MOV D A

INX H

JMP LOOP

DONE: SHLD 2054h

XCHG

SHLD 2056h

HLT

STOP: HLT

OUTPUT:

| ADDRESS | DATA |
| --- | --- |
| 2050 | 0A |
| 2051 | 00 |
| 2052 | 02 |
| 2053 | 00 |
| 2054 | 05 |

RESULT:

Thus, 16-bit multiplication and division were performed and verified successfully using the 8085 microprocessor.